From mrburns@shaw.wave.ca Sat Aug 29 11:40:59 1998

X-Delivered: at request of bin on uncle

Received: from norquay.tor.shaw.wave.ca (mail.tor.shaw.wave.ca [24.64.63.48])

by uncle.ssc.com (8.8.5/8.8.5) with ESMTP id LAA27421

for <gazette@ssc.com>; Sat, 29 Aug 1998 11:40:58 -0700

Received: from shaw.wave.ca ([24.64.133.185]) by norquay.tor.shaw.wave.ca

(Netscape Messaging Server 3.0)  with ESMTP id AAA2175

for <gazette@ssc.com>; Sat, 29 Aug 1998 14:42:01 -0400

Message-ID: <35E84CA3.AF0C7962@shaw.wave.ca>

Date: Sat, 29 Aug 1998 14:46:59 -0400

From: "Michael Burns2" <mrburns@shaw.wave.ca>

X-Mailer: Mozilla 4.05 [en] (Win95; I)

MIME-Version: 1.0

To: gazette@ssc.com

Subject: Re: megaraid

References: <199808291822.LAA02490@ripple.ssc.com>

Content-Type: text/plain; charset=iso-8859-1

Content-Transfer-Encoding: 8bit

Status: RO

no problem

start:

-------------------------

INSTALLING   THE   LINUX  MEGARAID   DRIVER

Ver: 0.91beta

Date: August 19, 1998

Requirements:

You need a full installation of Redhat 5.0, or Redhat 5.1, including the

kernel-headers and kernel-source packages.  If you don't have either of

these, you can get them off the CD or download them from Redhat, or

ftp.kernel.org.  The kernel version must be 2.0.x.  All kernels that

come with Redhat 5 are 2.0.x.  Support will be added soon for 2.1.x.

Applying the patch:

The patch file is named megaraid.patch.  You need to get this onto your

Linux system, and run it (in any directory).  If it stops and says "File

to patch:" then you don't have kernel-source or kernel-headers on the

system.  Go get them before you continue.

If you download the file directly onto your unix box, you need to make

sure megaraid.patch is executable before running it.  You can accomplish

this by typing:

chmod 755 megaraid.patch

If you get it onto the system using a DOS formatted diskette, use the

following commands to run the patch:

mount /dev/fd0 /mnt/floppy

/mnt/floppy/megaraid.pat

Now that the patch is applied, you can go on to rebuilding the kernel.

Recompiling your kernel:

Depending on your system configuration, you may want to customize your

kernel in any number of ways.  This document is only concerned with

adding MegaRAID support to an existing default kernel distribution.  The

following example will work with a default installation of Redhat, but

will need to be modified accordingly, if customized changes to the

system have been made.  More information on recompiling your kernel can

be found in the man pages and HOWTO documents for Linux.

All of the kernel compilation needs to be done from the /usr/src/linux

directory.  Type:

cd /usr/src/linux

To configure the kernel, type:

make menuconfig

A blue menusystem will come up.  To add MegaRAID support, select SCSI

support, then SCSI low-level drivers, and then AMI MegaRAID support.

Space bar will toggle between disabled, [ ], linked into the kernel [*],

and make as a module [M].

If you want to boot from a drive connected to a MegaRAID controller, you

will need to have it linked into the kernel.

Exit out of all menus, and then select Yes when it asks if you want to

save changes to the kernel configuration.

To compile the kernel, type:

make dep

make clean

make zImage

The last of the three commands will take a while, possibly more than 30

minutes, if you have a slow system.

When it's finished compiling, copy the kernel image generated into your

boot directory:

cp arch/i386/boot/zImage /boot/vmlinuz-2.0.31m

The name vmlinuz-2.0.31m is an example, and can be whatever you like.

If your kernel is not version 2.0.31, I would not recommend using this

example, as someone looking at your system later would be confused by

the different version number.  Don't name it the same as your old

kernel, which by default is /boot/vmlinuz-2.0.31 (or whatever the

version # is).

If you chose to compile the MegaRAID driver as a module (which can be

dynamically loaded and unloaded after the system has booted), you now

need to run:

make dep

make clean

make modules

make modules-install

Updating the OS loader:

The last step is to tell LILO (Linux loader) where to look for the

kernel image when booting up.  You need to edit the LILO configuration

file /etc/lilo.conf.  The old file should look something like this:

LILO.CONF   (OLD)   -----  unedited

boot=/dev/sda

map=/boot/map

install=/boot/boot.b

prompt

timeout=50

image=/boot/vmlinuz-2.0.31

label=linux

root=/dev/sda1

read-only

You need to create a new section, similar to the one starting with

image= and ending with read-only.  The new section should be added

before the old one, so that the default kernel for LILO to load will be

the new one.  The only lines that should be changed in the new section

should be image=<new kernel image from above> , and label=linux-mega.

The new file should look something like this:

LILO.CONF  (NEW)   -----   edited

boot=/dev/sda

map=/boot/map

install=/boot/boot.b

prompt

timeout=50

image=/boot/vmlinuz -  2.0.31m

label=linux-mega

root=/dev/sda1

read-only

image=/boot/vmlinuz-2.0.31

label=linux

root=/dev/sda1

read-only

If you want, you can change the image= to any name you want.  This is

the name that will appear when you presss TAB at the LILO prompt while

booting.

Before you reboot, make sure you run the following command:

lilo

This will write the new LILO configuration to the boot sector of the

hard drive, so the changes will take effect.

When you reboot, it should now automatically load the new kernel, and

should scan for a MegaRAID controller while booting.

If anything goes wrong and the new kernel doesn't load automatically,

press TAB and then select the label name of the old kernel ("linux", by

default).

SPECIAL  NOTES

1.   This  driver  supports   the   428 , 466   and  438  controllers.

2.   Currently  Redhat Linux  5.0  is   the  only Linux distribution

this   driver   has   been  tested  on.

3.   This   driver   has   not   been  tested   for   multi  processor

support,  or   any   extented

service   with-in   Linux  (only  basic  installation).   It

has   not   been   tested   in   a   Networking

environment.

4.  This   driver   has   not   been   tested   for   performance.

5.    Currently, this driver does not support multiple luns from the

upper layer.  Multiple logical drives will  appear as targets on a

channel 1 greater than the last channel on the card.

6.  This   is   a   AMI   beta   driver   which    is   currently

under   development.

7.  The   following   firmwares   have  been   tested   with   the

0.91   driver.

428  firmwares :     U79

438  firmwares :     1.24

466  firmwares :     2.09,  2.10

Driver  changes  since  last  release:

The  patch   command   has   been   added    to   the   source   code

to   ensure   compatibilty

with   Linux   5.1.

_______________________________________]

end:

Update to follow

Linux Gazette wrote:

>

> Thanks for the patches you sent. I will post them with LG issue32 in the

> Tips section. However the bottom part of your file "install.doc" is in

> Microsoft format and I can't read it. Could you resend this part in ascii

> so that I can post it along with the patches? My goal is to post on Monday

> but it may be Tuesday.

>

> Let me know.

>

> Thanks,

> Margie

> --

> -----------------------------------------------------------------------------

> Marjorie L. Richardson               |   Specialized Systems Consultants

> Managing Editor, Linux Journal       |   P.O. Box 55549

> Overseer & Editor, Linux Gazette     |   Seattle,WA 98155-0549

> -----------------------------------------------------------------------------